Capture DHCP&DNS Packets and Analysis

Name: Ou Yihang

Class: 2014215104

BUPT Student ID: 2014212948

QM Student ID: 140919433

- The configuration of Wireshark

- Using the command “scp –X student@192.168.56.101” to connect the physical machine and virtual machine.

- Using the command “sudo wireshark” then input the password to start the wireshark, then this will be show in the screen.

from this picture, there are 2 network is available.

Using the “ifconfig” to check the network status, then you could know the network “enp0s5” could use for capture packet.

- Capture DHCP packet

To capture DHCP packet, use “udp port 67” as capture filter, click “click”, use” bootp” to display filter.

- Capture DNS packet

To capture DNS packet, use “udp port 53” as capture filter, use “dns” to display filter.

- The result of DHCP capture and packet analysis

- Capture packet

After configure the wireshark, we start the DHCP packet capture.

Use the “sudo dhclient -r enp0s5”to kill old client process.

from this picture, we get the DHCP Release packet.

Use the “sudo dhclient enp0s5”to initial the DHCP connection.

From this picture, we get the 4 packets, DHCP Discover, DHCP offer, DHCP Request, DHCP ACK.

- Packet analysis

- DHCP release Packet

- Critical parameters and explanation

| Field | Parameter | Explanation |
| --- | --- | --- |
| Message type | Boot Request (1) | This is a request message. |
| Transaction ID | 0x25a33a43 | It’s used by the client to match responses with requests. |
| Hops | 0 | If the data packet transmission via router, each stand adds 1. If in the same network, then0 |
| Client IP address | 10.211.55.5 | The client has no IP address after release. This filed in only when the client definitely knows its IP addresses. |
| Your IP address | 0.0.0.0 | This is the IP address the server wants to allocate to the client and it’s filled by the server. |
| Server IP address | 0.0.0.0 | This is filled with server’s IP address when it sends DHCPOFFER, DHCPACK and DHCPNACK packets. |
| Router IP address | 0.0.0.0 | This is filled with relay agent’s IP address. |
| Option=53 | Length 1 | DHCP Message Type, Discover (1) |
| Option=54 | Length 4 | DHCP Server Identifier: 10.211.55.1 |
| Option=12 | Length 15 | Host Name: ubuntu |

- Address

|  | Frame address | IP Address | Port Number |
| --- | --- | --- | --- |
| Source | Parallel_19:bd:aa (00:1c:42:19:bd:aa), | 10.211.55.5 | 68 |
| Destination | Parallel_00:00:18 (00:1c:42:00:00:18) | 10.211.55.1 | 67 |

- DHCP Discover Packet

- Critical parameters and explanation

| Field | Parameter | Explanation |
| --- | --- | --- |
| Message type | Boot Request (1) | This is a request message. |
| Transaction ID | 0xb89d0603 | It’s used by the client to match responses with requests. |
| Hops | 0 | If the data packet transmission via router, each stand adds 1. If in the same network, then0 |
| Client IP address | 0.0.0.0 | The client has no IP address after release. This filed in only when the client definitely knows its IP addresses. |
| Your IP address | 0.0.0.0 | This is the IP address the server wants to allocate to the client and it’s filled by the server. |
| Server IP address | 0.0.0.0 | This is filled with server’s IP address when it sends DHCPOFFER, DHCPACK and DHCPNACK packets. |
| Router IP address | 0.0.0.0 | This is filled with relay agent’s IP address. |
| Option=53 | Length 1 | DHCP Message Type, Discover (1) |
| Option=50 | Length 4 | Requested IP Address: 10.211.55.5 |
| Option=12 | Length 15 | Host Name: ubuntu |
| Option=55 | Length 13 | Parameter Request List (e.g.: subnet mask, router, etc.) |

- Address

|  | Frame address | IP Address | Port Number |
| --- | --- | --- | --- |
| Source | Parallel_19:bd:aa (00:1c:42:19:bd:aa) | 0.0.0.0 | 68 |
| Destination | Broadcast (ff:ff:ff:ff:ff:ff) | 255.255.255.255 | 67 |

- DHCP Offer Packet

- Critical parameters and explanation

| Filed | Parameter | Explanation |
| --- | --- | --- |
| Message type | Boot Reply (2) | This is a request message. |
| Transaction ID | 0xb89d0603 | It’s used by the client to match responses with requests. |
| Hops | 0 | If the data packet transmission via router, each stand adds 1. If in the same network, then0 |
| Client IP address | 0.0.0.0 | The client has no IP address after release. This filed in only when the client definitely knows its IP addresses. |
| Your IP address | 10.211.55.5 | This is the IP address the server wants to allocate to the client and it’s filled by the server. |
| Server IP address | 10.211.55.1 | This is filled with server’s IP address when it sends DHCPOFFER, DHCPACK and DHCPNACK packets. |
| Router IP address | 0.0.0.0 | This is filled with relay agent’s IP address. |
| Option=53 | Length 1 | DHCP Message Type, Discover (1) |
| Option=54 | Length 4 | IP Address Lease Time: (1800s) 30 minutes |
| Option=51 | Length 15 | Host Name: ubuntu |
| Option=1 | Length 4 | Subnet Mask: 255.255.255.0 |
| Option=3 | Length 4 | Router: 10.211.55.1 |
| Option=6 | Length 4 | Domain Name Server: 10.211.55.1 |
| Option=15 | Length 11 | Domain Name: localdomain |
| Option=12 | Length 26 | Host Name: Ubuntu Linux 16.04 Desktop |

- Address

|  | Frame address | IP Address | Port Number |
| --- | --- | --- | --- |
| Source | Parallel_00:00:18 (00:1c:42:00:00:18) | 10.211.55.1 | 67 |
| Destination | Parallel_19:bd:aa (00:1c:42:19:bd:aa) | 10.211.55.5 | 68 |

- DHCP Request Packet

- Critical parameters and explanation

| Filed | Parameter | Explanation |
| --- | --- | --- |
| Message type | Boot Request (1) | This is a request message. |
| Transaction ID | 0xb89d0603 | It’s used by the client to match responses with requests. |
| Hops | 0 | If the data packet transmission via router, each stand adds 1. If in the same network, then0 |
| Client IP address | 0.0.0.0 | The client has no IP address after release. This filed in only when the client definitely knows its IP addresses. |
| Your IP address | 0.0.0.0 | This is the IP address the server wants to allocate to the client and it’s filled by the server. |
| Server IP address | 0.0.0.0 | This is filled with server’s IP address when it sends DHCPOFFER, DHCPACK and DHCPNACK packets. |
| Router IP address | 0.0.0.0 | This is filled with relay agent’s IP address. |
| Option=53 | Length 1 | DHCP Message Type, Discover (1) |
| Option=50 | Length 4 | Requested IP Address: 10.211.55.5 |
| Option=12 | Length 15 | Host Name: ubuntu |
| Option=55 | Length 13 | Parameter Request List (e.g.: subnet mask, router, etc.) |

- Address

|  | Frame address | IP Address | Port Number |
| --- | --- | --- | --- |
| Source | Parallel_19:bd:aa (00:1c:42:19:bb:aa) | 0.0.0.0 | 68 |
| Destination | Broadcast (ff:ff:ff:ff:ff:ff) | 255.255.255.255 | 67 |

- DHCP ACK Packet

- Critical parameters and explanation

| Filed | Parameter | Explanation |
| --- | --- | --- |
| Message type | Boot Reply (2) | This is a request message. |
| Transaction ID | 0xb89d0603 | It’s used by the client to match responses with requests. |
| Hops | 0 | If the data packet transmission via router, each stand adds 1. If in the same network, then0 |
| Client IP address | 0.0.0.0 | The client has no IP address after release. This filed in only when the client definitely knows its IP addresses. |
| Your IP address | 10.211.55.5 | This is the IP address the server wants to allocate to the client and it’s filled by the server. |
| Server IP address | 10.211.55.1 | This is filled with server’s IP address when it sends DHCPOFFER, DHCPACK and DHCPNACK packets. |
| Router IP address | 0.0.0.0 | This is filled with relay agent’s IP address. |
| Option=53 | Length 1 | DHCP Message Type, Discover (1) |
| Option=54 | Length 4 | IP Address Lease Time: (1800s) 30 minutes |
| Option=51 | Length 15 | Host Name: ubuntu |
| Option=1 | Length 4 | Subnet Mask: 255.255.255.0 |
| Option=3 | Length 4 | Router: 10.211.55.1 |
| Option=6 | Length 4 | Domain Name Server: 10.211.55.1 |
| Option=15 | Length 11 | Domain Name: localdomain |
| Option=12 | Length 26 | Host Name: Ubuntu Linux 16.04 Desktop |

- Address

|  | Frame address | IP Address | Port Number |
| --- | --- | --- | --- |
| Source | Parallel_00:00:18 (00:1c:42:00:00:18) | 10.211.55.1 | 67 |
| Destination | Parallel_19:bd:aa (00:1c:42:19:bd:aa) | 10.211.55.5 | 68 |

- The each DHCP message, unicast or broadcast?

- Frame No.1 (discover) : broadcast

- Frame No.2 (offer) : unicast

- Frame No.3 (request) : broadcast

- Frame No.4 (ACK) : unicast

- Compare the value of fields of DHCP messages with the example in Lecture note

- Frame No.1 (discover)

| My Messages |  |  |  |  |  | Example in lecture notes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  | 6 |  | 0 | 1 | 1 |  | 6 |  | 0 |
| 0xb89d0603 |  |  |  |  |  | 12 |  |  |  |  |  |
| 0 |  |  | Flags |  |  | 0 |  |  | Flags |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 00:1c:42:19:bd:aa |  |  |  |  |  | AA:EC:F9:23:44:19 |  |  |  |  |  |
| 53 |  | 1 |  | 1 |  | 53 |  | 1 |  | 1 |  |

Except for the transaction ID and the mac address, all fields are the same.

- Frame No.2 (Offer)

| My Message |  |  |  |  |  | Example in lecture notes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 |  | 6 |  | 0 | 2 | 1 |  | 6 |  | 0 |
| 0xb89d0603 |  |  |  |  |  | 12 |  |  |  |  |  |
| 0 |  |  | Flags |  |  | 0 |  |  | Flags |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 10.211.55.5 |  |  |  |  |  | 192.168.10.35 |  |  |  |  |  |
| 10.211.55.1 |  |  |  |  |  | 192.168.10.98 |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 00:1c:42:19:bd:aa |  |  |  |  |  | AA:EC:F9:23:44:19 |  |  |  |  |  |
| 53 |  | 1 |  | 2 |  | 53 |  | 1 |  | 2 |  |

Only the transaction ID, your IP address, the next server IP address and the mac address are not the same as the example in lecture notes.

- Frame No.3 (Request)

| My Message |  |  |  |  |  | Example in lecture notes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  | 6 |  | 0 | 1 | 1 |  | 6 |  | 0 |
| 12 |  |  |  |  |  | 12 |  |  |  |  |  |
| 0 |  |  | Flags |  |  | 0 |  |  | Flags |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 00:1c:42:19:bd:aa |  |  |  |  |  | AA:EC:F9:23:44:19 |  |  |  |  |  |
| 53 |  | 1 |  | 3 |  | 53 |  | 1 |  | 3 |  |

Except for the transaction ID and the mac address, all fields are the same.

- Frame No.4 (ACK)

| My Message |  |  |  |  |  | Example in lecture notes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 |  | 6 |  | 0 | 2 | 1 |  | 6 |  | 0 |
| 12 |  |  |  |  |  | 12 |  |  |  |  |  |
| 0 |  |  | Flags |  |  | 0 |  |  | Flags |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 10.211.55.5 |  |  |  |  |  | 192.168.10.35 |  |  |  |  |  |
| 10.211.55.1 |  |  |  |  |  | 192.168.10.98 |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| 00:1c:42:19:bd:aa |  |  |  |  |  | AA:EC:F9:23:44:19 |  |  |  |  |  |
| 53 |  | 1 |  | 5 |  | 53 |  | 1 |  | 5 |  |

Only the transaction ID, your IP address, the next server IP address and the mac address are not the same as the example in lecture notes.

- Message Sequence Chart (MSC)

- The result of DNS capture

- Capture Packet

- Packet analysis

- DNS Query Type=A

| Critical Parameter |  | Value | Explanation |
| --- | --- | --- | --- |
| Transaction |  | 0x472f | 16-bit field used to correlate queries and responses. |
| Parameters 0x0100 Standard Query | Response | 0… …. …. …. | 1-bit field that identifies the message as a query (0) or response (1). Message is a query. |
|  | Opcode | .000 0… …. …. | Standard query (name to address) |
|  | Truncated | …. ..0. …. …. | Message is not truncated. |
|  | Recursion desired | …. …1 …. …. | The resolver requests recursive service by the name server. |
|  | Z | ….. …. .0.. …. | Set to 0 for future use. |
|  | Non-authenticated data | …. …. …0 …. | Unacceptable. |
| Flag |  | 0x0100 | This is a message that the host send to server, so it is a quire. |
| Question section |  | 1 | The number of available question is 1 (the question is at the end of the message). |
| Answer section Authority section Additional section |  | 0 0 0 | These three are in answer section. This is a query message, so the three are all 0. |
| Frame Address |  | Parallel_19:bd:aa (00:1c:42:19:bd:aa) | Source MAC Address. |
|  |  | Parallel_00:00:18 (00:1c:42:00:00:18) | Destination MAC Address(Broadcast). |
| IP Address |  | 10.211.55.5 | Source IP Address. |
|  |  | 10.211.55.1 | Destination IP Address. |
| Port Number |  | 52656 | Source port |
|  |  | 53 | Destination port |

| Critical Parameter |  | Value | Explanation |
| --- | --- | --- | --- |
| Transaction |  | 0x472f | 16-bit field used to correlate queries and responses. |
| Parameters 0x0100 Standard Query | Response | 1… …. …. …. | 1-bit field that identifies the message as a query (0) or response (1). Message is a query. |
|  | Opcode | .000 0… …. …. | Standard query (name to address) |
|  | Truncated | …. ..0. …. …. | Message is not truncated. |
|  | Recursion desired | …. …1 …. …. | The resolver requests recursive service by the name server. |
|  | Recursion available | …. …. 1… …. | Server can do recursive queries |
|  | Z | ….. …. .0.. …. | Set to 0 for future use. |
|  | Anwser authenticated | …. …. ..0. …. | Answer/authority portion was not authenticated by the server |
|  | Non-authenticated Data | …. …. …0 …. | Unacceptable. |
|  | Reply code: | .... .... .... 0000 = | No error (0) |
| Flag |  | 0x8180 | This is a message that the host send to server, so it is a quire. |
| Question section |  | 1 | The number of available question is 1 (the question is at the end of the message). |
| Answer section |  | 3 | There are 3 IP Addresses for baidu server. |
| Authority section |  | 0 | No Authority section. |
| Additional section |  | 0 | No Additional section. |
| Frame Address |  | Parallel_00:00:18 (00:1c:42:00:00:18) | Source MAC Address. |
|  |  | Parallel_19:bd:aa (00:1c:42:19:bd:aa) | Destination MAC Address(Broadcast). |
| IP Address |  | 10.211.55.1 | Source IP Address. |
|  |  | 10.211.55.5 | Destination IP Address. |
| Port Number |  | 53 | Source port |
|  |  | 52656 | Destination port |

- DNS Query Response TYPE=MX

| Critical Parameter |  | Value | Explanation |
| --- | --- | --- | --- |
| Transaction |  | 0x096e | 16-bit field used to correlate queries and responses. |
| Parameters 0x0100 Standard Query | Response | 0… …. …. …. | 1-bit field that identifies the message as a query (0) or response (1). Message is a query. |
|  | Opcode | .000 0… …. …. | Standard query (name to address) |
|  | Truncated | …. ..0. …. …. | Message is not truncated. |
|  | Recursion desired | …. …1 …. …. | The resolver requests recursive service by the name server. |
|  | Z | ….. …. .0.. …. | Set to 0 for future use. |
|  | Non-authenticated data | …. …. …0 …. | Unacceptable. |
| Flag |  | 0x0100 | This is a message that the host send to server, so it is a quire. |
| Question section |  | 1 | The number of available question is 1 (the question is at the end of the message). |
| Answer section Authority section Additional section |  | 0 0 0 | These three are in answer section. This is a query message, so the three are all 0. |
| Frame Address |  | Parallel_19:bd:aa (00:1c:42:19:bd:aa) | Source MAC Address. |
|  |  | Parallel_00:00:18 (00:1c:42:00:00:18) | Destination MAC Address(Broadcast). |
| IP Address |  | 10.211.55.5 | Source IP Address. |
|  |  | 10.211.55.1 | Destination IP Address. |
| Port Number |  | 34428 | Source port |
|  |  | 53 | Destination port |

| Critical Parameter |  | Value | Explanation |
| --- | --- | --- | --- |
| Transaction |  | 0x096e | 16-bit field used to correlate queries and responses. |
| Parameters 0x0100 Standard Query | Response | 1… …. …. …. | 1-bit field that identifies the message as a query (0) or response (1). Message is a query. |
|  | Opcode | .000 0… …. …. | Standard query (name to address) |
|  | Truncated | …. ..0. …. …. | Message is not truncated. |
|  | Recursion desired | …. …1 …. …. | The resolver requests recursive service by the name server. |
|  | Recursion available | …. …. 1… …. | Server can do recursive queries |
|  | Z | ….. …. .0.. …. | Set to 0 for future use. |
|  | Anwser authenticated | …. …. ..0. …. | Answer/authority portion was not authenticated by the server |
|  | Non-authenticated Data | …. …. …0 …. | Unacceptable. |
|  | Reply code: | .... .... .... 0000 = | No error (0) |
| Flag |  | 0x8180 | This is a message that the host send to server, so it is a quire. |
| Question section |  | 1 | The number of available question is 1 (the question is at the end of the message). |
| Answer section |  | 4 | There are 4 IP Addresses for baidu server. |
| Authority section |  | 0 | No Authority section. |
| Additional section |  | 0 | No Additional section. |
| Frame Address |  | Parallel_00:00:18 (00:1c:42:00:00:18) | Source MAC Address. |
|  |  | Parallel_19:bd:aa (00:1c:42:19:bd:aa) | Destination MAC Address(Broadcast). |
| IP Address |  | 10.211.55.1 | Source IP Address. |
|  |  | 10.211.55.5 | Destination IP Address. |
| Port Number |  | 53 | Source port |
|  |  | 34428 | Destination port |

- DNS QUERY TYPE=PTR

| Critical Parameter |  | Value | Explanation |
| --- | --- | --- | --- |
| Transaction |  | 0x4b33 | 16-bit field used to correlate queries and responses. |
| Parameters 0x0100 Standard Query | Response | 0… …. …. …. | 1-bit field that identifies the message as a query (0) or response (1). Message is a query. |
|  | Opcode | .000 0… …. …. | Standard query (name to address) |
|  | Truncated | …. ..0. …. …. | Message is not truncated. |
|  | Recursion desired | …. …1 …. …. | The resolver requests recursive service by the name server. |
|  | Z | ….. …. .0.. …. | Set to 0 for future use. |
|  | Non-authenticated data | …. …. …0 …. | Unacceptable. |
| Flag |  | 0x0100 | This is a message that the host send to server, so it is a quire. |
| Question section |  | 1 | The number of available question is 1 (the question is at the end of the message). |
| Answer section Authority section Additional section |  | 0 0 0 | These three are in answer section. This is a query message, so the three are all 0. |
| Frame Address |  | Parallel_19:bd:aa (00:1c:42:19:bd:aa) | Source MAC Address. |
|  |  | Parallel_00:00:18 (00:1c:42:00:00:18) | Destination MAC Address(Broadcast). |
| IP Address |  | 10.211.55.5 | Source IP Address. |
|  |  | 10.211.55.1 | Destination IP Address. |
| Port Number |  | 34250 | Source port |
|  |  | 53 | Destination port |

| Critical Parameter |  | Value | Explanation |
| --- | --- | --- | --- |
| Transaction |  | 0x4b33 | 16-bit field used to correlate queries and responses. |
| Parameters 0x0100 Standard Query | Response | 1… …. …. …. | 1-bit field that identifies the message as a query (0) or response (1). Message is a query. |
|  | Opcode | .000 0… …. …. | Standard query (name to address) |
|  | Truncated | …. ..0. …. …. | Message is not truncated. |
|  | Recursion desired | …. …1 …. …. | The resolver requests recursive service by the name server. |
|  | Recursion available | …. …. 1… …. | Server can do recursive queries |
|  | Z | ….. …. .0.. …. | Set to 0 for future use. |
|  | Anwser authenticated | …. …. ..0. …. | Answer/authority portion was not authenticated by the server |
|  | Non-authenticated Data | …. …. …0 …. | Unacceptable. |
|  | Reply code: | .... .... .... 0000 = | No error (0) |
| Flag |  | 0x8180 | This is a message that the host send to server, so it is a quire. |
| Question section |  | 1 | The number of available question is 1 (the question is at the end of the message). |
| Answer section |  | 0 | There are 4 IP Addresses for baidu server. |
| Authority section |  | 0 | No Authority section. |
| Additional section |  | 0 | No Additional section. |
| Frame Address |  | Parallel_00:00:18 (00:1c:42:00:00:18) | Source MAC Address. |
|  |  | Parallel_19:bd:aa (00:1c:42:19:bd:aa) | Destination MAC Address(Broadcast). |
| IP Address |  | 10.211.55.1 | Source IP Address. |
|  |  | 10.211.55.5 | Destination IP Address. |
| Port Number |  | 53 | Source port |
|  |  | 34250 | Destination port |

- Compare the DNS message with the one in Lecture notes

- DNS Query TYPE=A

|  | My Messages | Example in lecture notes |
| --- | --- | --- |
| Header | Opcode=standard query | Opcode=standard query |
| Question | QNAME=www.baidu.com | QNAME=SRI-ARPA |
|  | QCLASS=IN | QCLASS=IN |
|  | QTYPE=A | QTYPE=A |
| Answer section | <empty> | <empty> |
| Authority section | <empty> | <empty> |
| Additional section | <empty> | <empty> |

- DNS Query Response TYPE=A

|  | My Messages | Example in lecture notes |
| --- | --- | --- |
| Header | Opcode=standard query | Opcode=standard query |
| Question | QNAME=www.baidu.com | QNAME=SRI-ARPA |
|  | QCLASS=IN | QCLASS=IN |
|  | QTYPE=A | QTYPE=A |
| Answer section | www.baidu.com www.a.shifen.com (IN A 220.181.112.244) | SRI-NIC.ARPA 86400 IN A 26.0.0.73 86400 IN A 10.0.0.51 |
| Authority section | <empty> | <empty> |
| Additional section | <empty> | <empty> |

- DNS Query TYPE=MX

|  | My Messages | Example in lecture notes |
| --- | --- | --- |
| Header | Opcode=standard query | Opcode=standard query |
| Question | QNAME=www.baidu.com | QNAME=SRI-ARPA |
|  | QCLASS=IN | QCLASS=IN |
|  | QTYPE=MX | QTYPE=MX |
| Answer section | <empty> | <empty> |
| Authority section | <empty> | <empty> |
| Additional section | <empty> | <empty> |

- DNS Query Response TYPE=MX

|  | My Messages | Example in lecture notes |
| --- | --- | --- |
| Header | Opcode=standard query | Opcode=standard query |
| Question | QNAME=www.baidu.com | QNAME=SRI-ARPA |
|  | QCLASS=IN | QCLASS=IN |
|  | QTYPE=MX | QTYPE=MX |
| Answer section | jpmx.baidu.com mx50.baidu.com mx1.baidu.com mx.n.shifen.com | SRI-NIC.ARPA 86400 IN A 26.0.0.73 86400 IN A 10.0.0.51 |
| Authority section | <empty> | <empty> |
| Additional section | <empty> | <empty> |

- DNS Query TYPE=PTR

|  | My Messages | Example in lecture notes |
| --- | --- | --- |
| Header | Opcode=standard query | Opcode=standard query |
| Question | QNAME=www.baidu.com | QNAME=SRI-ARPA |
|  | QCLASS=IN | QCLASS=IN |
|  | QTYPE=PTR | QTYPE=PTR |
| Answer section | <empty> | <empty> |
| Authority section | <empty> | <empty> |
| Additional section | <empty> | <empty> |

- DNS Query Response TYPE=PTR

|  | My Messages | Example in lecture notes |
| --- | --- | --- |
| Header | Opcode=standard query | Opcode=standard query |
| Question | QNAME=www.baidu.com | QNAME=SRI-ARPA |
|  | QCLASS=IN | QCLASS=IN |
|  | QTYPE=PTR | QTYPE=PTR |
| Answer section | www.a.shifen.com | SRI-NIC.ARPA 86400 IN A 26.0.0.73 86400 IN A 10.0.0.51 |
| Authority section | baidu_dns_master.baidu.com | mname |
| Additional section | <empty> | <empty> |

- Use nslookup to resolve type ”NS” ，“CNAME” query.

- Type=NS

- TYPE=CNAME